900M enterprise-level outdoor digital bridge

N620 is a high-performance enterprise-level outdoor bridge product that works in the 5G frequency band and supports 802.11ac technology. Unique digital tube pairing technology, without computer configuration, easily complete the pairing of point-to-point and point-to-multipoint (within 8 o'clock) devices. Gigabit + 100M network interface, 5G 802.11ac MIMO technology wireless processing speed up to 900Mbps. The power supply mode is flexible, supporting 24V POE network cable power supply and 12V 1A DC local power supply, and the network cable power supply distance can reach 50-70 meters (depending on the network cable material). It adopts outdoor IP65 windproof, rainproof, dustproof and sun protection grade shell design, which can easily adapt to various harsh outdoor environments. Built-in 14dBi dual-excited plate antenna, easy and quick installation. It has the characteristics of high performance, high gain, high receiving sensitivity, high bandwidth, etc., which greatly enhances the wireless transmission performance and stability, and is widely used in medium and short distance video transmission and data transmission.

Hardware product features

Super cost-effective hardware configuration

Enterprise-level main control chip, industrial-level circuit design, support IEEE802.11a/n/ac protocol, can provide 900Mbps wireless access speed and 1000Mbps+100Mbps Ethernet switching processing speed. The excellent anti-high and low temperature design fully guarantees the real-time, long-term, stable and high-efficiency transmission of user network data in extreme environments, and improves user experience.

Powerful wireless transmission capability

The circuit design with high power and high receiving sensitivity greatly enhances the wireless transmission distance and improves the performance and stability of wireless transmission.

Simple and efficient pairing

No need for network expertise, no need for computer operation, easily dial the code and adjust the master and slave device digital tubes to the same value to complete the point-to-point, point-to-multipoint (within 8 points) pairing work.

Flexible power supply

The equipment not only supports the power supply mode of POE remote network cable power supply, but also supports the power supply mode of 12V 1A DC local connection power supply, which can meet the needs of various scenarios, reduce construction costs, and flexibly select power supply methods.

Stylish and compact, easy to install

The appearance is stylish and compact, and the installation method is flexible and simple. It has a wall-mounted and pole-mounted fixed installation method. On the basis of not affecting the original design, it greatly reduces the difficulty of construction for construction personnel and improves construction efficiency.

Software features

Humanized unified management & settings

Built-in centralized controller function, wired or wireless connected devices can access 169.254.254.254 through the WEB browser, you can list all devices on the local area network, easily configure and wirelessly optimize the devices. Solve the problem that the LAN wireless bridge is difficult to manage and greatly simplify the work of the network administrator.

A clear overview of the equipment

Log in to the device through a WEB browser to view the connection system status, bridge status, and interface status information in real time. The simple and clear interface allows users to know the working status of the device in real time.

Fool-style quick setup & powerful wireless optimization

Users do not need to understand too much wireless knowledge, do not need too much professional knowledge, and can realize the pairing of point-to-point and point-to-multipoint (within 8 points) devices in a few simple steps. Simple settings can complete wireless optimization, and easily improve link transmission quality and use effect.

Protect users' network security at all times

The equipment bridge and VAP signal adopt advanced WPA-PSK&WPA2-PSK encryption strategy, and the bridge signal cannot be searched by wireless terminals. The VAP signal supports SSID hiding, which protects the user's network from hacker attacks at all times and ensures the user's data security.

Easily adapt to multiple network environments

The device supports gateway mode and bridge transparent transmission mode. The gateway mode supports automatic acquisition, PPPoE, and static IP, which can easily adapt to various network environment requirements.

Unique mobile management mode

The device supports 5G VAP signal transmission and HTML5 on the mobile phone. The connection signal of the mobile phone and tablet can be easily logged in 169.254.254.254 to configure the device.

Cloud management

Unique cloud management, as long as the network where the device is connected to the Internet, easy configuration can access the configuration device anywhere.

Simple and efficient system configuration function

Network time synchronization, access password modification, local backup/restore settings, factory reset, local/online upgrade, immediate/timed restart, simple and clear functions allow you to easily complete the system settings of the device and improve the robustness of the device.

Product technical specifications

| Product parameter |  |
| --- | --- |
| Hardware Configuration |  |
| Model | N620 |
| Main chip | MTK7620DA+7621E 900Mbps High-performance enterprise chip |
| Main frequency | 580MHz MIPS® 24KEc™ CPU |
| Wireless technology | 5G:900M 802.11a/n/ac MIMO Technology |
| Memory | 64MB DDR RAM |
| Flash | 8MB |
| Network Interface | WAN: 1*10/100/1000 Mbps adaptive RJ45 network interface LAN: 1*10/100Mbps adaptive RJ45 network interface |
| Button | 1*Digital switch/reset button, short press the digital tube to display the value plus one, long press for 15 seconds to restore factory settings |
| Indicator light | Signal SIG indicator, LAN1/LAN2 network interface status indicator, power indicator, digital tube indicator |
| Power supply | 24V 1A non-standard POE power supply; DC 12V 1A, power consumption <10W |
| Working environment | Temperature: -30℃～+55℃(work), -40℃～+70℃(storage) Humidity (non-condensing): 10% to 90% (working), 5% to 95% (storage) |
| Product Size | N/A |
| Product weight | N/A |
| Sky line | Built-in dual-polarization high-gain 14dBi directional plate antenna (half angle of horizontal wave 60°, half angle of vertical wave 60°) |

| RF characteristics |  |
| --- | --- |
| Frequency Range | ISM band: 4.900GHz ~ 5.850GHz |
| Channel distribution | 5G：36、40、44、48、52、56、60、64、149、153、157、161 |
| Modulation | OFDM = BPSK,QPSK,16-QAM,64-QAM，256-QAM;DSSS = DBPSK,DQPSK,CCK |
| Output Power | 11a @54M:20±2dB, @6M:23±2Db 11n 20MHz：@MCS7:23±2dB, @MCS0:23±2dB 11n 40MHz：@MCS7:20±2dB, @MCS0:23±2dB 11ac 40MHz @MCS7:20±2dB, @MCS0:23±2dB 11ac 80MHz @MCS7:20±2dB, @MCS0:23±2Db |
| Receiving sensitivity | 11a: -65dbm@54Mbps, -81dbm@6Mbps 11n 20MHz: -64dbm@MCS7, -82dbm@MCS0 11ac 40MHz：-61dbm@MCS7, -79dbm@MCS0 11ac 80MHz: -58dBm@MCS7 -76dBm@MCS0 |
| EVM | 802.11n: ≤-28 dB 802.11a: ≤-25 dB |
| Frequency deviation | ＜±20ppm |

| Software features |  |
| --- | --- |
| Operating mode | Master AP, slave AP (dial code conversion) |
| Networking method | Point-to-point, point-to-multipoint (within 8 points) |
| Management method | Chinese WEB remote management |
| Internet | Bridge: static IP/dynamic acquisition Gateway: static IP/dynamic acquisition/PPPoE |
| Bridge management | Free binding cloud account, real-time remote management of equipment |
| Wireless management | Bridge configuration/advanced configuration/WiFi settings |
| System | Time zone time configuration, login password modification, backup/restore, restart/timed restart |
| System update | Local update and online update based on WEB browser |